[Date]

Faisal A

[Company name]

[Company address]

Simple Robotic Arm Prototype

Contents

1. Project Summary	2

2. Problem and Objectives	2

2.1. Problem Statement	2

2. 2. Project Objectives	2

3. System Overview	2

3.1 Mechanical Architecture	2

3.2 End Effector (Gripper)	3

4. Electronics and Control	3

4.1 Actuation and Hardware	3

4.2 Control Strategy	3

4.3 Simulation	4

5. Design Iterations	4

5.1 Initial Concept	4

5.2 Refined Design	4

6. Testing and Results	4

6.1.	4

6.2. Iteration 1 – Initial Concept	5

6.1 Test Scenario	5

6.2 Observed Performance	5

6.3 Limitations Identified	5

7. Key Engineering Takeaways	5

8. Future Improvements	5

9. Conclusion	6

# 1. Project Summary

This project involved the design and construction of a low-cost robotic arm prototype to explore fundamental robotic system design, servo-based actuation, and Arduino control. Built using inexpensive materials and minimal tooling, the prototype was intended as a proof-of-concept learning platform rather than a high-performance manipulator.

The robotic arm successfully demonstrated basic pick-and-place functionality for lightweight objects, providing hands-on insight into mechanical layout, control logic, and the practical limitations of rapid prototyping with low-cost materials.

# 2. Problem and Objectives

## 2.1. Problem Statement

Design a simple robotic arm capable of picking up a lightweight object, repositioning it through horizontal and vertical motion, and releasing it at another location, using minimal materials and an Arduino-based control system.

## 2. 2. Project Objectives

- Build a functional robotic arm using low-cost, readily available materials

- Implement multi-joint servo control using an Arduino UNO

- Validate mechanical and control concepts through physical testing

- Identify design limitations and improvement opportunities for future iterations

# 3. System Overview

## 3.1 Mechanical Architecture

- Total arm length: ~20 cm

- Link lengths: ~10 cm per segment

- Degrees of freedom:

  - Base rotation (horizontal motion)

  - Arm rotation (vertical motion)

  - Servo-driven gripper (open/close)

The arm was constructed using wooden sticks, cardboard, string, and craft glue. All joints were servo-actuated revolute joints. The base was not externally mounted, which resulted in instability under load and required manual stabilization during operation.

## 3.2 End Effector (Gripper)

A simple pincer-style gripper was used to manipulate objects. The final design employed:

- One servo-driven movable claw

- One stationary claw

- Rubber band friction surfaces to improve grip

This configuration reduced complexity and improved reliability compared to an earlier dual-motor gripper concept.

prototype.

# 4. Electronics and Control

## 4.1 Actuation and Hardware

Controller: Arduino UNO R3

Motors: SG90 9g servo motors

Power: Arduino 5 V rail

No power brownout or overheating issues were observed during testing, though the configuration is recognized as suitable only for low-load operation.

## 4.2 Control Strategy

Open-loop servo position control (no feedback sensors)

Pushbuttons used for continuous joint rotation while pressed

Slide switch used for discrete gripper open/close control

Servo angle limits were tuned to prevent over-rotation and mechanical binding, with final gripper limits set to approximately 50° (open) and 120° (closed).

## 4.3 Simulation

The circuit and control logic were first developed and tested in TinkerCAD, allowing verification of motor behavior and input logic prior to physical assembly.

# 5. Design Iterations

## 5.1 Initial Concept

- Four servo motors: two for arm motion, two for gripper claws

- Button-based control for all joints

- Increased wiring complexity and limited breadboard space

## 5.2 Refined Design

- Reduced to three servo motors

- Simplified gripper with a single actuated claw

- Replaced gripper buttons with a slide switch to conserve I/O space

- Removed unnecessary linkages to improve stability and weight distribution

- These changes improved usability and robustness without sacrificing required functionality.

# 6. Testing and Results

6.1. Degrees of Freedom and Motion
The arm was designed with two degrees of freedoms and a gripper:

- DoF 1: Base rotation (horizontal motion)

- DoF 2: Arm rotation (vertical motion)

- Gripper: Opening and closing motion (excluded from DoF count)

At minimum, two DoF were required to reposition the end effector in space and complete a pick-and-place task.

## 6.2. Iteration 1 – Initial Concept

## 6.1 Test Scenario

- Object: crumpled paper

- Task: pick up object, reposition it, and release it

## 6.2 Observed Performance

- Horizontal and vertical motions were repeatable within visual tolerance

- Gripper successfully grasped and released lightweight objects

- Motion smoothness was limited by structural flex and servo mounting rigidity

## 6.3 Limitations Identified

- Fragile construction due to craft glue joints

- Base instability caused by arm weight and torque

- Limited dexterity due to lack of a wrist joint

Despite these limitations, the prototype consistently completed the intended pick-and-place task.

# 7. Key Engineering Takeaways

- Early-stage robotic prototypes benefit greatly from mechanical rigidity, even at low loads

- Simplifying actuation and control often improves reliability more than adding complexity

- Simulation tools are effective for validating control logic before hardware assembly

- Low-cost materials are sufficient for validating system-level concepts

# 8. Future Improvements

- Replace structural elements with 3D-printed components

- Use servo mounting brackets and hot glue or fasteners for improved rigidity

- Implement potentiometer or joystick-based proportional control

- Add additional degrees of freedom, such as a wrist joint

- Redesign the gripper with interchangeable or compliant tips

# 9. Conclusion

This robotic arm prototype met its objective as a learning-focused proof-of-concept system. While structurally limited, it successfully demonstrated multi-joint servo control, basic manipulation, and iterative design decision-making. The insights gained from this project directly inform future robotic arm designs with improved materials, refined control strategies, and enhanced mechanical performance.